# GreenWave High Resolution (Advanced) Log Decoding Information

This document provides information on the High Resolution (Advanced) Log Events based on firmware version 3.32. Table 1 lists all Events and their corresponding Data Parameters. Table 2 lists the definition of all Data Parameters used in Table 1. Table 3 lists the enumerated values for Events that use History Value and New Value for their Data Parameters. The last section explains how to decode a Log file.

## Event IDs

Table 1 – GreenWave Advanced Log Events

| Event ID | Event Name and Data Parameters |
| --- | --- |
| 0 | Phase On/Off = of |
| 1 | Phase Next On/Off = of |
| 2 | Phase Check On/Off = of |
| 3 | Phase Green Begin = p# |
| 4 | Phase Yel Warn Begin = p# |
| 5 | Phase Yellow Begin = p# |
| 6 | Phase Red Begin = p# |
| 7 | Phase Pedestrian Walk Begin = p# |
| 8 | Phase Pedestrian Clearance (FDW) Begin = p# |
| 9 | Phase Pedestrian Solid Don’t Walk (SDW) Begin = p# |
| 10 | Phase Hold On/Off = of |
| 11 | Phase Omit On/Off = of |
| 12 | Phase Force-Off On/Off = of |
| 13 | Pedestrian Omit On/Off = of |
| 14 | Vehicle Detector Call On/Off = of |
| 15 | Pedestrian Detector Call On/Off = of |
| 16 | Phase Minimum Green Complete = p# |
| 17 | Phase Yellow Warning Complete = p# |
| 18 | Phase Yellow Complete = p# |
| 19 | Phase Red Clearance Complete = p# |
| 20 | Phase Termination by Gap-Out = p# |
| 21 | Phase Termination by Max-Out = p# |
| 22 | Phase Termination by Force-Off = p# |
| 23 | Phase Termination by Interval Advance = p# |
| 24 | Phase Termination by Red Rest = p# |
| 25 | Vehicle Detector On/Off = of |
| 26 | Pedestrian Detector On/Off = of |
| 27 | Detector Gap Disconnect = d# |
| 28 | Volume/Occupancy Sequence Change = v |
| 29 | Reserved Event |
| 30 | Vehicle Detector Occupancy Change = d#, v |
| 31 | Overlap On/Off = of |
| 32 | Overlap State Change = o#, hv, nv – See Table 3 for hv and nv values |
| 33 | Overlap Flash on/off = of |
| 34 | Pedestrian Overlap Walk Begin = po# |
| 35 | Pedestrian Overlap Pedestrian Clearance (FDW) Begin = po# |
| 36 | Pedestrian Overlap Solid Don’t Walk (SDW) Begin = po# |
| 37 | Preemption Input On/Off = of |
| 38 | Preemption Gate Down Input On/Off = of |
| 39 | Preemption State Change = pr#, hv, nv – See Table 3 for hv and nv values |
| 40 | Preemption Link On/Off = of |
| 41 | Preemption Maximum Presence = pr# |
| 42 | NTCIP Pattern Status Change = hv, nv |
| 43 | Free Status Change = pt#, v |
| 44 | Coordination Status Change = pt#, hv, nv – See Table 3 for hv and nv values |
| 45 | Critical Intersection Control (CIC) Status Change = pt#, hv, nv – See Table 3 for hv and nv values |
| 46 | Cycle Length Change = pt#, nv |
| 47 | Offset Length Change = pt#, nv |
| 48 | Split Change = pt#, s#, nv |
| 49 | Coordination Ring Yield = r#, p# |
| 50 | Pattern Change = hv, nv |
| 51 | Unit Control Status Change = hv, nv – See Table 3 for hv and nv values |
| 52 | Flash Status Change = hv, nv – See Table 3 for hv and nv values |
| 53 | Alarm Status 1 Change = hv, nv – See Table 3 for hv and nv values |
| 54 | Alarm Status 2 Change = hv, nv – See Table 3 for hv and nv values |
| 55 | Short Alarm Status Change = hv,nv – See Table 3 for hv and nv values |
| 56 | Special Function On/Off = of |
| 57 | Barrier Crossed = hv, nv |
| 58 | Vehicle Detector Alarm Change = d#, hv, nv – See Table 3 for hv and nv values |
| 59 | Pedestrian Detector Alarm Change = d#, hv, nv – See Table 3 for hv and nv values |
| 60 | Vehicle Detector Reported Alarm (RAlarm) Change = d#, hv, nv – See Table 3 for hv and nv values |
| 61 | Channel Red On/Off = of |
| 62 | Channel Yellow On/Off = of |
| 63 | Channel Green On/Off = of |
| 64 | Power Interrupt On/Off = of |
| 65 | Manual Control Enable (MCE) On/Off = of |
| 66 | Interval Advance (I/A) On/Off = of |
| 67 | Test Input On/Off = ti# |
| 68 | Alarm Group Change = hv, nv – See Table 3 for hv and nv values |
| 69 | Controller Unit (CU) Alarm State Change = hv, nv – See Table 3 for hv and nv values |
| 70 | Date Update = D, M, Y |
| 71 | Time Update = h, m, s |
| 72 | Transit Priority (TSP) Check-In Input On/Off = of |
| 73 | Transit Signal Priority (TSP) Check-Out Input On/Off = of |
| 74 | Transit Signal Priority (TSP) State Change = hv, nv – See Table 3 for hv and nv values |
| 75 | Transit Signal Priority (TSP) Input Change State = rn#, hv, nv – See Table 3 for hv and nv values |
| 76 | Texas Diamond Transition State Change = hv, nv – See Table 3 for hv and nv values |
| 77 | Texas Diamond Current Mode Change = hv, nv – See Table 3 for hv and nv values |
| 78 | Texas Diamond Command Mode Change = hv, nv – See Table 3 for hv and nv values |
| 79 | TSP Remote Check-In Request = rn#,B1,B2,B3,B4,B5,B6,R1,R2,R3,R4,R5,R6,R7 |
| 80 | TSP Remote Check-Out Request = rn#,B1,B2,B3,B4,B5,B6,R1,R2,R3,R4,R5,R6,R7 |
| 81 | Preemption Remote Check-In Request =pr# |
| 82 | Preemption Remote Check-Out Request =pr# |
| 83 | Cam Queue Len change = ca#,z#,hv,nv |
| 84 | Local Data Slot Chng = sl#,hv,nv |
| 85 | TSPRmt ChkInReq Data = B1,B2,B3,B4,B5,B6,l,T,eta,mx,A |
| 86 | TSP Q Jump Active = rn# |
| 87 | TSP Q Jump Result = rn#,qr |
| 88 | TSP Split Adjustment = rn#,p#,t2,+ |
| 89 | TSPRmt ChkIn = rn#,B1,B2,B3,B4,B5,B6 |
| 90 | TSPRmt ChkOut = rn#,B1,B2,B3,B4,B5,B6,f |
| 91 | TSPRmt Run Number = B1,B2,B3,B4,B5,B6,R1,R2,R3,R4,R5,R6,R7,R8,R9 |
| 92 | Veh Det Vol Change = d#,v |
| 93 | TSP Service Delay = rn#,s1 |
| 94 | TSP ChkIn Data = rn#,c,e |
| 95 | TSP Phase/Ped Omit = rn#,p#,p |
| 96 | TSP Ph Shift/Reserve = rn#,p#,r |
| 97 | Coord Local Zero = <nothing> |
| 98 | Phase Skipped = p#,sc,pg |
| 99 | Phase Serviced = p#,sc,pg,ts,rs,ge,gr,sa,d |
| 100 | Startup Pattern = pt#,fr |
| 101 | Free New Cycle = <nothing> |
| 102 | Ph Extn Timer GapOut = p# |
| 103 | Ph Gap Reduce Start = p# |
| 104 | Ph Min Gap Attained = p# |
| 105 | Ph Preempt Force Off = p#,pr# |
| 106 | Ovlp FYA on/off = of |
| 107 | Ph Added Init Complt = p# |
| 108 | Ph V Det call on/off = of |
| 109 | Ph P Det call on/off = of |
| 110 | Ph Maximum Mode = p#,x |
| 111 | Ph DynMax Step Up/Dn = dm |
| 112 | Ph Adv Warn on/off = of |
| 113 | Ovlp Adv Warn on/off = of |
| 114 | Stop Time on/off = of |
| 115 | Coord Phase Begin = p# |

## Event Decoding Parameters

Table 2 – GreenWave Advanced Log Data Parameter Descriptions

| Data Parameter | Data Parameter Definition (all sizes are 1 byte unless otherwise indicated) |
| --- | --- |
| of | On/Off Bit 0: 0=off, 1=on Bits 1-7: the instance number (i.e., Phase Number). Bits 1-7 are 0 if the event is global (i.e., MCE Event ID 65) |
| p# | Phase Number (1 – 16) |
| d# | Detector Number (1 – 64) |
| v | Value |
| hv | Historical Value – see Table 3 for events with enumerated values |
| nv | New Value – see Table 3 for events with enumerated values |
| o# | Overlap Number (1 – 32) |
| po# | Pedestrian Overlap Number (1 – 16) |
| pr# | Preemption Run Number (1 – 32) |
| pt# | NTCIP Pattern Number (1 – 255) |
| s# | Coordination Split Table Number (1 – 32) |
| r# | Ring Number (1 – 4) |
| c# | Channel Number (1 – 32) |
| ti# | Test Input Number (1=A, 2=B, 3=C, etc.) |
| D | Day |
| M | Month |
| Y | Year |
| h | Hour |
| m | Minute |
| s | Second |
| rn# | Run Number |
| Bx | Bus ID Byte x = (1 – 6) (data is ASCII char code) |
| Rx | Route ID Byte x = (1 - 7) (data is ASCII char code) |
| ca# | External Camera Number (1 – 8) |
| z# | External Camera Zone Number (1 – 100) |
| sl# | Local Data Slot Number (1 – 16) |
| l | Log-Only TSP or Preempt Remove Request 0=Log and take action 1=Log but take no action |
| T | Time in epoch 0 to 0xFFFFFFFF (4 bytes) Seconds since Jan 1, 1970 00:00:00 |
| eta | Estimated Time of Arrival in seconds from most recent check in (2 bytes) |
| mx | Remove TSP or Preempt Call maximum active time seconds (2 bytes) |
| qr | TSP Queue Jump result 0=success 1=max-out/failure |
| s1 | Generic time in whole seconds 0 – 255 |
| s2 | Generic time in whole seconds 0 – 65535 (2 bytes) |
| t1 | Generic time in tenth seconds 0 – 255 |
| t2 | Generic time in tenth seconds 0 – 65535 (2 bytes) |
| + | Split Adjustment (0=reduced, 1=extended) |
| d | Phase Service Delay time in seconds (0 – 255) |
| f | Force Off Code 0=not forced off 1=forced off |
| fr | Free/Coord (0=Coord, 1=Free) |
| A | TSP Database Enabled 0=TSP disabled by database disable or no TSP Action Plan in effect 1=TSP enabled |
| c | TSP Input Mode 0=Check-In/Out pulses 1=Constant Call mode |
| e | Generic Enable/disable code (0=disabled, 1=enabled) |
| p | Ped/Veh Omit 0=ped and veh omitted 1=only ped omitted |
| r | Shift Re-service code 0=standard TSP Shift no re-service 1=TSP shift with re-service. |
| x | Maximum Mode 1=max 1 2=max 2 3=dynamic max 4=free pattern max (pattern cycle time set to zero) |
| dm | Dynamic Max Step Up/Down Bit 0: 0=step down, 1=step up Bits 1-7: Phase Number |

| sc | Service/Skip Code 0=no demand or phase omit 1=TSP skip 2=TSP EAT skip 3=no ped demand 4=TSP ped skip | 5=TSP EAT ped skip 6=veh and ped served 7=veh and oversized ped served 8=dual entry 9=conditional service |
| --- | --- | --- |
| pg | Programmed Max Green (if Free) or Green Split (if Coord) in tenth seconds (2 bytes) |  |
| ts | Termination Status 0=gap 1=max 2=fixed force off 3=floating force off 4=forward force off |  |
| rs | Relative Split Start Time (3 bytes) Byte 1: 0=started late, 1=started early or on time Bytes 2 and 3: amount of time the Phase Split started early/late in tenth seconds |  |
| ge | Accumulated Green Extension time in tenth seconds (2 bytes) |  |
| gr | Accumulated Green Rest time in tenth seconds (2 bytes) |  |
| sa | Split Adjustment Code 0=no split adjustment 1=extended via offset correction not due to TSP 2=reduced via offset correction not due to TSP 3=extended via offset correction after TSP service 4=reduced via offset correction after TSP service 5=extended via offset correction to sync CIC timings 6=reduced via offset correction to sync CIC timings 7=extended via TSP green extension applied to standard Pattern 8=reduced via TSP green reduction applied to standard Pattern 9=extended via TSP green extension applied to CIC timings 10=reduction via TSP green reduction applied to CIC timings 11=extended via CIC Split time larger than the Programmed Split time 12=reduced via CIC Split time smaller than the Programmed Split time 13=preemption |  |

## Event values for new / historical type events

Table 3 – GreenWave Advanced Log Enumerated Events Using History value and New value

| Event ID | Enumerated Values for hv and nv |  |  |
| --- | --- | --- | --- |
| 32 | 0=dark 1=leading green 2=delay green 3=green | 4=yellow 5=red 6=trail green 7=yellow warning |  |
| 39 | 0=idle 1=start 2=dwell phase start 3=dwell flash 4=exit flash all-red 5=dwell phase | 6=waiting for all-red 7=dwell all-red 8=entry 9=track green 10=exit start 11=exit |  |
| 44 | 0=free 1=in sync 2=transition add 3=transition reduce | 4=transition dwell 5=local zero 6=pickup cycle 7=master zero |  |
| 45 | 0=idle 1=pending active 2=active 3=error 4=pending idle (waiting for local zero) |  |  |
| 51 | NTCIP unitControlStatus - see NTCIP 1202 mib obj for values |  |  |
| 52 | NTCIP unitFlashStatus - see NTCIP 1202 mib obj for values |  |  |
| 53 | NTCIP unitAlarmStatus1 - see NTCIP 1202 mib for bit definitions |  |  |
| 54 | NTCIP unitAlarmStatus2 - see NTCIP 1202 mib for bit definitions |  |  |
| 55 | NTCIP shortAlarmStatus - see NTCIP 1202 mib for bit definitions |  |  |
| 58 | NTCIP vehicleDetectorAlarms - see NTCIP 1202 mib for bit definitions |  |  |
| 59 | NTCIP pedestrianDetectorAlarms - see NTCIP 1202 mib for bit definitions |  |  |
| 60 | NTCIP vehicleDetectorReportedAlarms - see NTCIP 1202 mib for bit definitions |  |  |
| 68 | NTCIP alarmGroupState - see NTCIP 1202 mib for bit definitions |  |  |
| 69 | NTCIP controllerAlarmStatus1 byte value changed. Byte is encoded as: Bit 0 = TSP Active Bit 1 = CIC Active (Critical Intersection Control) Bit 2 = UPS Communications Link Failure Bit 3 = 2010 CMU Communications Link Failure Bit 4 = Time Source Switched to Time Clock Chip (instead of AC Line) Bit 5 = Reserved Bit 6 = NTCIP Communications active Bit 7 = USB installed Bit 8 = ATC Cabinet Flasher Alarm |  |  |
| 74 | 0=TSP offset correction done waiting for local zero 1=idle 2=TSP active 3=standard TSP recovery 4=split balance TSP recovery |  |  |
| 75 | NTCIP tspInputStatus 0=None 1=Request 2=Re-service Lock Out 3=Success 4=Removed 5=Clearance Fail 6=Detector Fail |  | 7=Detector Fail Cleared 8=reserved for future 9=Delay Timing 10=Extend Timing 11=Preempt Override 12=Split Extending 13=Invalid Config 14=Clearance Success |
| 76 | 0=no transition 1=in transition |  |  |
| 77 | 0=no diamond 1=four phase 2=three phase 3=nema 4=separate |  |  |
| 78 |  |  |  |

# Decoding a Log File

A log file (binary) is created automatically by GreenWave for each hour when that hour ends.

The filename indicates the month, day, year and hour with the following format:

YYYY_MM_DD_HH.dat.gz

These files can be retrieved by inserting a USB drive into the Controller and selecting from the USB Menu “8 Log Menu” then “3 Advance CU Log”. The files will be stored in the USB at the path

<USB root>/ATC_LINUX/USTC_logs/ADVCULOGS/<Central MAC Address>/<Log Files>

Example:

I:/ATC_LINUX/USTC_logs/ADVCULOGS/70-B3-D5-50-11-86/2021_08_26_19.dat.gz

The .gz file must be unzipped first to YYYY_MM_DD_HH.dat

The beginning of each .dat file contains a Header encoded as human-readable ASCII bytes as follows:

Line 1: ATC Data Collection Format Version <ver>\n

Line 2: YYYY_MM_DD_HH.dat\n

Line 3: IP Address aaa.bbb.ccc.ddd\n

Line 4: MAC Address aa:bb:cc:dd:ee:ff\n

Line 5: Begin(hh:mm:ss:tt) HH:MM:SS:TT\n

Line 6: Phases in use: <phase numbers separated with comma>\n

Line 7: Specialty Log Count: 0\n

Line 8: Binary Data Follows:\n

After “Binary Data Follows:” plus newline character 0x0A (\n), the event data bytes start.

The binary data contains events one after the other (until the end of file indicator) with the following format for each event:

1 byte for event ID #

2 bytes for the tenth-second count for this hour

Variable # of bytes for the event

To get the parsing info for a given event ID number, refer to Tables 1, 2 and 3 above.

Example Decoding Binary Log File Data on a File with Only 2 events:

Line 1: ATC Data Collection Format Version 102\n

Line 2: 2021_04_11_10.dat\n

Line 3: IP Address 010.247.001.128\n

Line 4: MAC Address 00:50:60:70:80:90\n

Line 5: Begin(hh:mm:ss:tt) 10:00:00:00\n

Line 6: Phases in use: 1,2,3,4,5,6,7,8,9,10\n

Line 7: Specialty Log Count: 0\n

Line 8: Binary Data Follows:\n

Binary data (hex): 2C 00 01 06 07 01 0B 00 09 0B

0x2C = 44. Table 1 indicates Event ID 44 is “Coord Status Change” with Data Descriptors pt#, hv, nv.

According to Table 2, these Data Descriptors are defined as:

pt# = pattern number (1 to 255), 1 byte

hv  = history value, 1 byte

nv  = new value, 1 byte

According to Table 3, the enumerated values of the hv and nv for event 44 are:

0=free, 1=in sync, 2=trans add, 3=trans reduce, 4=trans dwell, 5=local zero, 6=pickup cycle, 7=master zero

Therefore, the complete decoding of the remaining bytes for this event is:

00 01 = tenth-second count 1

06 = pattern number 6 running

07 = previous Coord Status was “master zero”

01 = new Coord Status is “in sync”

0x0B = 11. Table 1 indicates Event ID 11 is “Phase Omit on/off” with Data Descriptor of.

According to Table 2, this Data Descriptor is defined as:

Bit 0: 0=off, 1=on

Bits 1-7: the instance number (i.e., Phase Number).

Bits 1-7 are 0 if the event is global (i.e., MCE Event ID 65)

Therefore, the complete decoding of the remaining bytes for this event is:

00 09 = tenth-second count 9

0B = 1011 binary.

First bit is 1, which means ON, and 101 is the phase number, phase 5, which means phase 5 omit turned on.